# Telecoms - Transport Layer

=> Logical communication between app processes.

=> Run in end systems

- Send side: breaks message into segments, passed to network layer

- Rcv side: reassembles segments into message, passes to app layer

Multiplexing/Demultiplexing

Multiplexing @ sender

=> Handles data from multiple sockets

=> Adds transport header (used for demultiplexing)

Demultiplexing @ recv

=> Uses header to deliver rcvd segs into correct sockets

=> Host recv’s datagram:

- Contains header

  - TCP socket identified by src adr, dst adr, src port, dst port (4 tuple)

  - UDP socket identified by dst adr, dst port3

- Each segment has its own dst, src port to direct segment to correct socket to support multiple sockets

- Webservers different socket for each client

UDP

=> Barebones: simple transport protocol, connectionless, small header

=> Best efforts: segs may be lost, delivered out of order, no congestion control

=> Connectionless: no handshaking between sndr/recv, segs handled independently

=> Used: for streaming multimedia (loss tolerant/rate sensitive), DNS

=> Header:

RDT (Reliable data transfer)

=> We want to give the illusion of a reliable channel

=> Characteristics of the realistic unreliable channel will determine complexity of protocol

=> Separate FSMs for sndr/recv, although both sides may send or recieve

RDT 1.0 (Reliable channel)

=> No bit errors / no packet loss

=> Circle = current state		|	Event

Dashed arrow = starting state	|	----------

Arrow = next state due to event 	|	Actions

RDT 2.0 (Channel has bit errors)

=> Includes error detection & feedback

=> Use checksum to detect errors

=> ACKs/NAKs used for error recover

=> Cannot deal with corrupted ACK/NAK

RDT 2.1 (Handles corrupted ACKs/NAKs)

Sender:

=> Seq number added to pkt (1/0)

=> Checks if ACK/NAK corrupted

Reciever:

=> Checks if pkt is duplicate

=> Cannot tell if ACKs/NAKs received by sender

RDT 2.2 (NAK free)

=> Same functionality as 2.1

=> Instead of NAK, receiver sends ACK for last OK pkt received

Bhy => Must include seq numbers of pkts being ACKd

=> Duplicate ACKs at sender results in same action as NAK (retransmit current pkt)

RDT 3.0 (Channel with errors and losses)

=> Channel can lose packets (data + ACKs)

=> Approach:

- Sender waits a set time (countdown timer required)

- Retransmits if no ACK recv in time

- If pkt/ACK just delayed (not lost):

  - Retransmission will be duplicate, but seq numbers handle this

  - Recv must specify seq number of the pkt being ACKd

Pipelined protocols

=> Allows for multiple “in-flight” pkts, so the RTT won’t affect the throughput as much,

As we don’t have to wait on ACKs (increased bandwidth utilisation)

=> Range of seq numbers must be increased rather than 0/1

=> Allows for buffering at sender/receiver

Go-back-N

=> Sender can have N unACKd pkts in pipeline

=> Recv can send cumulative ACKs (only sending the ACK for last pkt recv)

=> Sender has timer for oldest unACKd pkt

=> When timer expires, all unACKd pkts are retransmitted (retransmit current + all higher seq number pkts in window)

=> Seq number in pkt header

=> Out of order pkts discarded, reACK packet with highest seq number

Sender FSM

Recv FSM

Selective Repeat

=> Recv ACKs all individual OK packets

=> Packets are buffered to achieve in-order delivery for the upper layer

=> Sender only retransmits pkts for which ACK not recv’d

=> Need a timer for each packet to do this

=> Sender:

- If data sent from above ->

  - if next available seq# in window, send pkt

- Timeout(seq# n) ->

  - resend pkt n, restart timer

- Recv ack(seq# n) where seq# n is in window ->

  - mark pkt n as recv’d

  - if seq# n is the lowest seq# in window, advance window by 1

=> Recv:

- Pkt seq# n in recv window ->

  - Send ack(n)

  - If pkt out of order then buffer

  - If pkt in order, deliver (and deliver buffered out of order pkts)

    - Advance window to next not yet recv’d pkt

- Pkt seq# before recv window ->

  - ack(n) previously ack’d pkt

- Otherwise -> Ignore

TCP

=> Point to point (1 sender, 1 recv)

=> Reliable, in order byte stream (no message boundaries)

=> Pipelined (TCP congestion control/flow control set window size)

=> Full duplex data

- Bi directional data flow in same connection

- MSS (max seg size) e.g. 1500 bytes for ethernet

- MTU (max transmission unit) - 40 bytes (TCP/IP header)

=> Connection oriented

- Handshaking (exchange of control msgs)

- Initialising sender/receiver state before data exchange

=> Flow controlled

- Sender will not overwhelm receiver

TCP Header

=> Seq# = number of first byte in segment

=> ACK# = seq# of next byte expected from other side

TCP Round Trip Time

=> Timeout value

- Should be longer than RTT, but RTT varies

- Too short -> unnecessary retransmissions

- Too long -> slow reaction to segment loss

=> Estimating RTT

- Use sampleRTT (the time it takes from seg transmission to ACK receipt)

- SampleRTT varies due to load and congestion

=> Use EstimatedRTT

- (1 - ⍺) * EstimatedRTT + ⍺ * SampleRTT

- ⍺ usually 0.125 (between 0-1)

  - If too small -> sudden change in network not reflected fast enough

  - If too large -> transient fluctuations affect EstimatedRTT makes it unstable

  - Both lead to under/over EstimatedRTT

- Exponential weighted moving average

=> DevRTT

- TimeoutInterval = EstimatedRTT + 4*DevRTT (safety margin)

- DevRTT = (1-β * DevRTT + β*abs(SampleRTT - EstimatedRTT)

- Β usually 0.25

=> TimeoutInterval starts at 1, doubled when timeout occurs

TCP reliable data transfer

=> Pipelined segments

=> Cumulative ACKs

=> Single retransmission timer

=> TCP Sender

- Data recv’d from app above, create seg with seq# of 1st data byte in segment

- Start timer if not already running (timer for oldest unACKd pkt (expires after TimeoutInterval))

- If timeout occurs, retransmit seg that caused timeout + restart timer

- ACK recv’d

  - If ACK acknowledges previously unACKd segments, updated what is known to be ACKd and start timer if there are still unACKd segments

Sender:

TCP ACK generation (at receiver)

Event -> Action

=> arrival of in-order segs, with expected seq#, all data up to expected seq# ACKd ->

Send delayed ACK 500ms for next seg, if no next seg send ACK

=> arrival of in-order segs, with expected seq#, one other seg has ACK pending ->

Immediately send cumulative ACK, ACKing both in-order segs

=> arrival of out-of-order seg, with higher than expected seq#, gap detected ->

Immediately send duplicate ACK, with seq# of next expected byte

=> arrival of segment that fills a gap (partially or fully) ->

Immediately send ACK, provided seg starts at lower end of gap

TCP Fast Retransmit

=> Long timeout period

=> Lost segs detected by duplicate ACKs

=> If 3 duplicate ACKs, resend seg with smallest seq#

=> Likely that unACKd seg was lost, so don't wait for timeout

TCP Flow Control

=> Receiver controls sender, so sender won't overflow recv buffer by transmitting too much too fast, prevents buffer from overflowing

=> Recv advertises free buffer space

=> By using rwnd (recv window) value included in header of recv-to-sender segs

=> RcvBuffer size set via socket options, auto-adjusted by OS

Rwnd = RcvBuf - [ lastByteRcvd - lastByteRead ]

TCP Connections

=> Send/recv handshake, establishes connection

=> Agree on connection params

=> 3 way handshake

Establishing Connection

Closing Connection

SYN flood could (DOS)

- Large number of TCP SYN segs sent to server without completing step 3 of handshake

- Use SYN cookies to ensure legitimate user

- Server creates initial sequence number (ISN) from hash of Src adr/port, Dst adr/port, timestamp

- Server sends SYNACK

- Legit user returns and ACK, using cookie info (ISN+1) in ACK

Congestion Control

=> Too many sources sending too much data too fast for network to handle

=> Causes lost packets(buffer overflow), long delays(queuing in buffers)

=> 2 Approaches

- End-to-end(TCP)

  - No explicit feedback

  - Congestion inferred from end-system observed loss&delay

- Network Assisted

  - Routers provide feedback to end systems

  - Single bit indicating congestion (ATM)

ATM ABR

ABR available bit rate

=> Senders path underloaded -> sender uses more bandwidth

=>Senders path Congested -> sender throttled to minimum guaranteed rate

=>RM Cells (resource management)

- Sent by sender, between data cells

- Bits in rm cells set by switches (network assisted) -> NI/CI bits -> mild congestion/congestion

- Returned to sender by receiver

AIMD (additive increase, multiplicative decrease)

=> Sender increases transmission rate (window size)

=> Probing for usable bandwidth until loss occurs

=> Increase congestion window (cwnd) by 1 mss every rtt until loss detected

=> Cut cwnd in half after loss

=> Saw tooth behaviour

TCP Congestion Control

Slow Start

=> Rate increases exponentially until 1st loss occurs (starts at cwnd=1mss)

=> Double cwnd every RTT (incrementing cwnd for every ACK received)

=> Loss indicated by timeout, cwnd set to 1 mss

=> Window grows exponentially to ssthresh, then grows linearly

TCP Reno

- 3 duplicate ACKs indicates loss

- Cwnd cut in half, then grows linearly

TCP Tahoe

- Always sets cwnd to 1

- Timeout or 3 duplicate ACKs

=> Exponential switches to linear when cwnd set to ½ of its value before timeout

=> Variable ssthresh

# Telecoms - Application Layer

Client-Server Architecture

=> Server

- Always on host

- Permanent IP adr

=> Client

- Communicates with server

- Intermittent connection

- May have dynamic IP adr

- Do not communicate directly with each other

P2P peer to peer architecture

=> No always on server

=> End systems directly communicate

=> Peers request service from other peers, provide service in return to other peers

=> Self scalability -> new peers bring new service capacity, as well as demands

=> Peers are intermittently connected, change IP adrs, complex management

Processes

=> Client process -> inits communication

=> Server process -> waits to be contacted

=> Communication by exchanging messages

=> p2p architecture applications contain both client&server processes

Application’s Transport Services Needs

=> Data loss

- File transfer / transactions 100% reliable data transfer

- Audio / video tolerates loss

=> Timing -> telephony / games are time sensitive

=> Throughput -> multimedia needs minimum amount of throughput

=> Security -> authentication / encryption

WWW

=> URL (uniform resource locator) allows access to each object on webpage

[hostname]/[pathname]

HTTP

=> Uses TCP

- Client inits TCP connection (creates socket) to server @ port 80

- Server accepts connection

- HTTP msgs exchanged

- Connection closed

=> HTTP is stateless as TCP retains information.

=> Non-persistent HTTP

- 1 object sent over TCP connection, then connection closed

- 1 RTT to init TCP connection

- 1 RTT for HTTP request + first few bytes of HTTP response to return

- File transmission time

- Response time = 2*RTT + file transmission time

- Overhead for each TCP connection

- Browsers can open parallel TCP connections

=> Persistent HTTP (default with pipelining)

- Multiple objects send over single TCP connection

- Server leaves connection open after sending response

- Subsequent client/server messages sent over open connection

HTTP Request Message

=> Either request or response

=> PUT

- Uploads file to path in url field

=> DELETE

- Deletes file at path in url field

=> POST

- Input uploaded to server

=> GET

- Input is uploaded in URL field

HTTP Response Codes

=> Occur on 1st line of response message

=> 200 OK, 301 Moved Permanently, 400 Bad Request, 404 Not Found, 505 Version Not Supported

Cookies

=> Used for authorization, shopping carts, user session state

=> As HTTP is stateless, cookies may be used to maintain state at sender/receiver

Web Caches (Proxy server)

=> Want to satisfy request without involving origin server

=> Browser sends HTTP requests to proxy

=> If object in cache -> proxy returns object

=> Otherwise -> proxy requests object from origin server, returns obj to client

=> Cache acts as client&server

=> Caches reduce traffic on ISP, reduces response time for client request

=> Conditional GET used (if-modified-since:<date> in HTTP request), if cache has up-to-date version, send it with “304 Not Modified” response

=> If the cache’s version is out of date, send response from origin server

Email

=> User Agent

- Compose, edit, read messages

=> Mail Server

- Contains incoming messages, queue of outgoing messages

- SMTP protocol used between mail servers

=> SMTP

- Uses TCP, port 25

- Command/response interaction

- Uses persistent connections

- All messages (header&body) to be sent in 7 bit ASCII

- Both HTTP and STMP have ASCII command/response interaction, status codes

Multipurpose Internet Mail Extension (MIME)

=> Used to convert non ASCII data to ASCII to be sent via SMTP

=> Different MIME types for different data types (e.g. text, audio, image)

POP3 vs IMAP

POP3

=> Download and delete mode, cannot re-read email if client is changed

=> Download and keep, copies of messages on different clients

=> Stateless across sessions

IMAP

=> Keeps all messages at server

=> Allows user to organise messages in folders

=> Keeps user state across sessions

Domain Name System (DNS)

=> Distributed db implemented in hierarchy of many name servers

=> Hosts, name servers communicate to resolve names (adr/name translation)

=> Services:

- Hostname to IP adr translation

- Host aliasing

- Canonical, alias names

- Mail server aliasing

- Load distribution

- Replicated web servers

- Many IP adrs correspond to 1 name

=> Root servers contacted by local name server that cannot resolve name

=> Returns list of IP adrs for responsible TLD servers

=> Top level domain (TLD) servers, responsible for .com, .org etc

Local DNS

=> Does not strictly belong in the hierarchy

=> Each ISP has one

=> When host names DNS query, its sent to local DNS server

- Has cache of recent name to adr translation pairs (may be out of date)

- Acts as proxy, forwards query into hierarchy

DNS Name Resolution

DNS Name Resolution & Caching

DNS Records

=> Distributed db storing resource records(RR)

=> A: IPv4, AAAA: IPv6 (128 bit)

- NS: name server

  - Name is domain, value is hostname

- CNAME: canonical name (alias)

  - Name is alias for real name, value is real name

- MX: mail exchange

  - Value is name of mail server

DNS Inserting records

=> Register name at DNS registrar

=> Registrar creates 2 RRs into .com TLD server

- example.com, NS, dns1.example.com

- Dns1.example.com, A, 212.212.212.1

=> Create

- Authoritative server type A record for example.com

- MX type record for example.com

DNS Attacks

=> DDos -> bombard root servers with traffic, avoided as local DNS servers cache IPs, bypassing root

=> Redirect attacks (man in the middle - intercepting queries, DNS poisoning)

P2P vs Client Server - File Distribution

=> How long does it take to distribute file (size F) from one server to N peers?

Client-server:

=> Time to upload N copies = N*(F / us)

=> Time to download on a client = F/dmin

Where dmin  is the min client d/l rate

P2P

=> Server must upload at least one copy

=> Clients download file copy

=> Max upload rate = us + sum(ui)

BitTorrent

=> A torrent is a group of peers exchanging chunks of a file

=> File divided into 256kb chunks

=> Peers can send/recv chunks

=> Trackers track peers participating in torrent

=> Peer joins torrent

- No chunks yet, accumulates them over time from other peers

- Registers with tracker to get list of peers, connects to some peers (neighbours)

- Peers upload chunks to other peers

- Peers may change

- Once peer has entire file, may leave or remain in torrent

=> Peers will have different subsets of file chunks

=> Periodically, peers will ask each other for chunks that they have

=> The missing chunks are requested from other peers

=> Tit-for-tat, each peer has top 4 peers currently sending its chunks at highest rate

=> Top 4 re-evaluated every 10 secs

=> Other peers may be choked, so every 30 secs select a random peer & send chunks

Synchronous vs Asynchronous Web Communication

Synchronous (most common)

=> User must wait while new pages load

=> While request is being processed on the server, user cannot interact with the client

Asynchronous

=> Used in rich internet applications (RIAs)

=> Dynamic websites

=> Ajax (asynchronous XML and javascript)

- Download data from server in background

- Webpage doesn’t need to be refreshed

- User may interact with webpage while data loads

Typical Ajax Request

=> User clicks, invoking an event handler

=> Handler creates and XMLHttpRequest object

=> XHR object requests page from server

=> Server retrieves and sends data

=> XHR object fires an event when the object arrives (callback)

=> Callback event handler processes the data and displays it

=> Resources must be on the same domain (same origin policy)

Websockets

=> Two-way socket connection between web browser and server

=> Ajax has to poll server for data, HTTP connection overhead creates bottleneck

=> Websockets allow client side js to open and persist a connection to a server

=> Full duplex capability allows messages to be pushed to client from server

=> Without the overhead associated with traditional HTTP requests

=> Means that both parties can start sending data at any time

=> Websocket handshake establishes connection

=> Data is transferred as messages

=> Messages are framed, with 2 byte overhead per frame, reduces amount of non-payload data that is transferred, reductions in latency